# VOLUME 3

# TECHNICAL SPECIFICATIONS GENERAL

# VOLUME 3

# Section 1

# CONSTRUCTION SITE

## FENCING AND SIGNBOARDS AT THE CONSTRUCTION SITE

The Contractor must maintain the security of its activities, including fencing of the construction site according to the regulatory requirements;

- The Contractor shall install temporary billboard and permanent plaque at the main site in line with the of the Interreg VI-A IPA Hungary-Serbia Programme manual and in agreement with the Contracting Authority after Provisional Acceptance.

- Construction site board prepared in accordance with the Law on Planning and Construction shall be placed on the temporary fence adjacent to the entrance gate to the site;

- The Contractor shall provide the whole information concerning the regulations and procedures governing the use of local facilities for access, transport, storage facilities and in compliance with them to take measures for providing the necessary documents;

- The Contractor shall be aware of existing restrictions and shall be responsible for their observance during construction;

- The Contractor shall be liable for all damages on the existing infrastructure caused by them - they shall be repaired at its expense;

- The Contractor will be responsible for ensuring the control of any access or the right to leave the boundaries of the construction site, so that it does not lead to interference with the locals or damage to public or private property as a result of the entry or exit of its employees and subcontractors;

- The Contractor shall indemnify and hold harmless the Contracting Authority against any accusations arising from its failure to comply with the above point, including legal fees and costs.

## TEMPORARY SITE FACILITIES

- The Contractor, prior to the start of construction works, shall submit a draft Design for the organisation and execution of construction. The Design must be submitted no later than 15 days before the planned start of construction works. The design must indicate the work zones, as well as areas for temporary storage of necessary construction materials and goods, and areas for temporary settlements for the personnel of the Contractor and Supervisor.

- The Contractor shall provide and install all necessary facilities/installations for accommodation of its staff, including dressing and rest containers, toilets, drinking and washing water, electricity, etc. All costs for temporary facilities shall be included in the Bid.

## FACILITIES FOR THE CONTRACTOR AND SUPERVISOR

- The Contractor shall hand over the fully equipped office to the Supervisors within 2 weeks of being ordered to do so.

- The cost of office and accommodation shall be paid by the Contractor and shall be included in the unit prices in the Bill of Quantities.

- The Contractor shall procure, at its own risk and expense, all additional facilities outside the site that may be necessary for its work.

## OFFICES FOR THE SUPERVISOR

All offices for the Supervisor shall have at least two grounded electrical sockets, rooms exceeding 10 m2 floor area, having at least one additional socket per 5 m2 of floor area or part thereof.

The Contractor shall supply, install and maintain in the offices, equipment and furniture which shall be new, undamaged and complete with all necessary keys.

The Contractor shall supply, install and maintain furniture such as desks, cupboards, drawing tables and plan chests, chairs and shelves, etc. in the numbers, trademarks and quality as approved by the Beneficiary and the Supervisor.

The Contractor shall arrange internet connection.

## FACILITIES FOR THE CONTRACTOR

The Contractor shall provide and maintain on site suitable site offices for its own use. It shall also provide and maintain on approved sites, sufficient stores, tanks and workshops for the proper storage of materials and equipment.

The stores shall be of such size and construction to provide adequate storage and protection of stocks of material, fuel, spares, etc. in quantities ensuring uninterrupted progress of the work. The Contractor shall allow in its rates for all costs related to provision of the offices and workshops for its own use.

## SITE CLEANING

The Contractor shall make every effort to keep the site tidy and in orderly manner and to take at any time every possible precaution against the contamination of subsoil and groundwater.

The Contractor shall be responsible for making all arrangements for the disposal of solid and liquid wastes from the site. Furthermore, it shall give strict instructions to all its employees to use the sanitary accommodation provided at the site.

## STORAGE OF EQUIPMENT AND MATERIALS IN PUBLIC SPACE

Construction materials and equipment shall not be stored outside the site borders.

Where Works are to be completed in public spaces, all plant and excess material shall be removed immediately from the site upon completion of the relevant task so as to limit public objections and complaints.

## TRAFFIC ARRANGEMENTS

The Contractor shall as far as required, comply with all requirements and recommendations of the Police and Authorities regarding traffic arrangements and road safety measures on public roads outside the construction sites.

The Contractor shall, where necessary, provide all barriers and traffic signs agreed by the Supervisor.

Traffic diversions, if necessary, shall be planned and arranged with the responsible Authorities by the Contractor and harmonized with the Supervisor. No diversion shall be implemented without a written consent of the responsible Authority and after given information to the Supervisor. Access to the site shall be available to vehicles of emergency services and residents in the areas.

All traffic signs and traffic control signals, as necessary and/or may be required by the Police Authority for the safe direction and control of the traffic shall be provided, placed and maintained by the Contractor on the appropriate sites and locations on the access to the sites. The location and size of all such signs and the lettering thereon shall be agreed by the Supervisor before placement of the signs.

The Contractor shall reposition, cover or remove signs as required during the progress of the works.

If it is necessary to close the street, and temporarily divert traffic, then it is necessary to develop a Temporary Traffic Management Plan or Design. This project must be submitted 15 days before the closure of the street to the Police as well as the relevant Secretariat.

# CONTRACTOR`S GENERAL RESPONSIBILITIES

## MANAGEMENT OF THE PROJECT BY THE CONTRACTOR

- The Contractor shall provide the Quality Assurance Plan (QAP) for the management and execution of construction works;

- The QAP should reflect the management structure and clearly describe the duties, responsibilities and powers of each member of the Contractors' staff;

- The representative of the Contractor and its staff must possess experience and qualifications according to the contract, RS Law and type and scope of works;

- This QAP will be updated and provided again whenever there is a change in personnel.

## APPROVAL AND INSTRUCTION BY THE SUPERVISOR

Where reference is made in this Specification, the Bills of Quantities or in the drawings for approval, instruction or direction, they shall be given by the Supervisor.

Approvals, instructions or directions by the Supervisor shall not relieve the Contractor from its liabilities and responsibilities under the Contract.

## QUALITY ASSURANCE PLAN

- The Contractor shall be responsible for assuring such quality of materials, works and processes that shall comply with the requirements of the Specifications.

- In order to meet the specified requirements, the Contractor shall implement Quality Assurance System presented in Quality Assurance Plan containing the following details:

- Quality control procedures;

- Personnel responsibilities;

- Procurement procedures;

- Testing procedures;

- Equipment and measurement devices;

- Frequency of testing, measurements etc.;

- Holding points in production for inspection;

- Rejection and corrective procedures;

- Documentation and communication

- H&S and Environmental Plan.

- The Contractor shall be liable to keep a register of all materials delivered on site or implemented in the construction to be accessed for review upon request by the Supervisor or Contracting Authority. Also, the Contractor shall maintain archive of the whole correspondence and instructions.

The Contractor shall within 28 days of the date of the Letter of Acceptance provide the Supervisor with the Organisation chart containing names, CVs and duties of all key personnel whether or not they are related to quality assurance directly.

## WORK PROGRAMME FORM OF SUBMISSIONS

The Work Programme presented by the Contractor shall consist of a detailed schedule of all construction works and phases. Once approved, the Work Programme shall be binding for the construction works on site.

### REQUIREMENTS

The Contractor shall present a Work Schedule for execution of the works with distribution of resources and manpower, including volume of works, number of workers for the stage, coordination of activities, interaction with different participants in the process, time limit for execution and sequence of the works all in accordance with Contract.

### WORK PROGRAMME

Pursuant to the requirements, the Work Programme to be submitted by the Contractor shall show the planned monthly rates of progress between the programme dates for commencement and completion of each major item or work for the various stages of construction, in accordance with the Conditions of Contract.

The Work Programme shall take into account climatic conditions and other conditions, to ensure the completion of the works by the Contractor or other contractors, in accordance with the Contract.

The Contractor shall not be permitted to commence any construction work on that part of the works until the Supervisor has no objection to the method statements, drawings and calculations. Sufficient time for approval of drawings materials and method statements must be allowed for in the Work Programme for each component.

The Contractor shall allow in its Programme a reasonable period for work to be carried out by Public Utility Services, Authorities and the Beneficiary where necessary. The Beneficiary will provide all necessary assistance in liaising with such Authorities.

The Contractor shall also allow in its Programme sufficient time required for provisional acceptance and for the maintenance periods (Defects Notification Period) as stipulated in the Contract.

## MONTHLY PROGRESS REPORTS

During of the execution of the Contract, the Contractor shall follow the progress of activities relative to the time schedule and shall submit to the Supervisor Monthly reports for the results of its activities, conforming to the following requirements:

- The Report to be provided to the Supervisor in 1 hardcopy in Serbian and English languages as well as digitally (on CD enclosed to the Report);

- Diagrams with detailed progress description, Contractor’s documents, delivery, construction works, assembly and tests;

- Digital photos (on CD enclosed to the Report);

- Linear chart (schedules) for the current Stage, showing the actual and the planned progress;

- Provision of resources - actual and planned;

- Diagram for labour flow - actual and planned;

- Report, reflecting all considerable differences from the construction programme, and if necessary, explanation for the proposed steps to be undertaken for the completion of the approved programme;

- Statistics on safety and environment protection;

- Financial Statement.

When actual work progress differs from that shown in the Construction Programme, the Contractor shall submit an updated schedule to the Supervisor. The updated time schedule shall be current to the last day of a calendar month and shall show the detailed “work-as-executed” programme in respect of work carried out. They shall be submitted within ten working days of the following month at the latest.

## PROGRESS PHOTOGRAPHS

Digital colour photographs showing the progress of the Works in detail shall be taken by the Contractor every week, from positions to be selected by the Supervisor.

The Contractor shall hand over the corresponding electronic files to the Supervisor on a CD, as well as an electronic list numbering and labelling each photograph (location, date when taken and a brief description or title).

## CONTRACTOR’S DOCUMENTATION GENERAL

- For design, works and supply use of metric units is compulsory.

- All documents will be issued in English. Official documents, which are to be presented to state or municipality authorities, will be also issued in Serbian.

- Works documentation (see Chapter 3.1.4.) will be in English and in Serbian, except Construction Log, which will be in Serbian.

- Reports and correspondence documentation will be in English and in Serbian.

- When submitted as computer files the documents shall be under Windows, compatible with following formats: texts in MS Word, Tables in MS Excel, drawings in ACAD, time schedules in MS Project.

### WORKS DOCUMENTATION

The Contractor shall be liable to provide the Supervisor with due documentation as per local Regulations. The Contractor shall keep/maintain the following Works documentation:

- Inspection Book (forms laid down by the Law of the Republic of Serbia),

- Construction Log (forms laid down by the Law of the Republic of Serbia),

- Measurement Book (forms laid down by the Law of the Republic of Serbia),

- All necessary certificates (for material, equipment and other) during the works execution.

The Works Site Manager shall keep the Construction Log and submit the Measurement Book sheets of the executed works along with each invoice. The Measurement Book has to be verified by the Supervisor.

The Construction Manager has to enter the following data into the Construction Log:

- Number and qualification of workers executing the works,

- Number and type of construction machinery used for works execution,

- Weather conditions under which the works are executed,

- How the works are executed and if there is any deviation from the design, contract and regulations in doing so.

### AS-BUILT DESIGN

- Based on a survey of executed works, as-built technical documentation will be prepared by the Contractor and delivered to the Supervisor (in accordance with Law on Construction and Planning).

- As-built drawings for all executed works must be delivered in digital form and 3 (three) printed and bind hard copies, signed by the Responsible Designer and the Supervisor.

- Contractor is obliged to support the Beneficiary during technical acceptance and make necessary corrections in the submitted documentation and designs upon the request of Authorities in order to obtain a use permit.

# HEALTH & SAFETY AND ENVIRONMENT PROTECTION

## HEALTH & SAFETY

### GENERAL REQUIREMENTS

Without limiting the Contractor's obligations under the Conditions of Contract, the Contractor shall take all measures and precautions necessary to ensure the health, safety and welfare of staff, labour, and other persons authorised to be on the Site, as well as visitors and third parties. The Contractor shall prepare H&S Plan and develop detailed sequence and safety measures in the Organisational plan for the management and execution of the works.

The Contractor shall:

- Fully comply with the Law on Safety and Health at Work.

- Appoint a member of staff responsible for all matters related to health and safety for the duration of the Contract according to RS regulations.

- Provide and maintain equipment in a safe working condition and adopt safe methods of work.

- Adopt methods for the use, handling, storage, transport, and disposal of materials, and substances which are not injurious to health and safety.

- Provide and maintain adequate lighting, signing, and fencing of the Works.

- Provide adequate protective clothing and safety equipment, including such information, instruction, training and supervision as are necessary to ensure the health and safety of all persons employed on or entering on the Site in connection with the Works.

Safety equipment shall include but not be limited to:

- safety helmets,

- protective footwear with integral steel toe-caps,

- safety glasses, welding goggles and other eye protectors,

- ear defenders,

- safety harnesses,

- high visibility reflective vests,

- Fire extinguishers.

- Provide and maintain access to all places on the Site in a condition that is safe and without risk of injury.

- Provide and maintain adequate water, waste water and waste collection, for all offices, worksites, and laboratories erected on the Site.

- Provide and maintain adequate sanitary units at locations where works are in progress.

- Appoint a member of its staff to be responsible for the safety of the Works throughout any shutdown period and notify the Supervisor of the name and contact telephone number of the responsible person.

- Report all accidents to the Supervisor and appropriate authorities at the time of occurrence or as soon as possible thereafter.

### TESTING AND CERTIFICATION OF MECHANIZATION AND EQUIPMENT

- The Contractor shall provide and maintain equipment for lifting, embedding and transporting materials and must comply with all relevant requirements of the standards in Serbia;

- All equipment must be regularly maintained in accordance with the recommendations and standards of the manufacturer, according to local laws and recommendations of the relevant authority;

- The Contractor shall prepare and update a register of certificates of testing of the equipment used on construction sites according to RS Law;

- The Contractor must appoint competent personnel responsible for the operation of all kinds of equipment. They must provide evidence that they have passed training and have respective license for operating the specific equipment;

- All the technological equipment (with test certificates) used on or around the site must be equipped with the necessary protective devices that will be in continuous readiness;

- Should the Supervisor consider the Contractor’s method of working unsafe or that there are insufficient or inadequate safety barriers or other devices or that there is insufficient safety or rescue equipment, the Contractor shall change its method of working or install or strengthen safety and rescue equipment if so instructed. Such instructions shall not relieve the Contractor of any of its responsibilities under the Contract.

- The Contractor shall immediately notify the Supervisor about any accident that occurs, whether on site or off site, in which the Contractor is directly involved, and which resulted in any injury to any person whether directly concerned with the site or a third party. Such initial notification may be verbal and shall be followed by a written comprehensive report within 24 hours of the accident.

- Transportation of any material by the Contractor shall be in suitable vehicles, which do not cause spillage when loaded, and all loads shall be suitably secured. Any vehicle shall be removed from the site, which does not comply with this requirement or any of the local traffic regulations and laws.

- The Contractor shall ensure access to sites at all times to any authorised external institutes or experts carrying out safety inspections.

### FIRE PROTECTION

During the performance of the Contract the Contractor shall make arrangements to the agreement of the Supervisor for the protection of the permanent works and any temporary works and any adjacent property from fire and, if required, it shall give the Fire Authority access to all facilities periodically to inspect the fire prevention arrangements.

Particular care must be exercised in connection with the operation of electric arc welding equipment, oxyacetylene cutting equipment and other processes involving the use of naked lights. Special arrangements will be necessary for the storage of highly flammable liquids on the site.

The Contractor shall remove all waste and material of a flammable nature and take other steps as the Supervisor may require but this shall not relieve the Contractor of any of its obligations under the Contract.

## ENVIRONMENT PROTECTION

### GENERAL

- The Contractor shall take all necessary measures and precautions and otherwise ensure that the execution of the Works and all associated operations on or off site are carried out in conformity with statutory and regulatory environmental requirements.

- The Contractor shall take all measures and precautions to avoid any nuisance or disturbance arising from the execution of the Works. This shall be achieved wherever possible by suppression of the nuisance at source rather than abatement of the nuisance once generated.

- The provisions of these Sub-Clauses shall only be disregarded in respect of emergency work required for the saving of life or property or the safety of the Works.

- In the event of any spoil or debris or silt from the Sites being deposited on any adjacent land, the Contractor shall immediately remove all such spoil debris or silt and restore the affected area to its original state to the agreement of the Supervisor.

The offer should include appropriate cost-effective mitigation measures, which should form part of the project cost.

Environmental Management Plan (EMP) shall be prepared by the Contractor incorporating proposals concerning the implementation, management and monitoring of the environmental components of the project.

Within two (2) weeks from the commencement of the works, the Contractor shall submit an EMP with operational details of its proposals to the Supervisor for approval.

### ENVIRONMENTAL PROTECTION DURING CONSTRUCTION PERIOD

The Contractor shall use such construction methods and shall maintain all works so as to assure the stability and safety of the construction site and building in general.

The Supervisor has the power to disallow the methods of construction and/or the use of any borrow/stockpile/spoil disposal area if in their opinion the stability and safety of the Works or any adjacent features are in danger.

Following the construction works, the Contractor shall take all steps necessary to complete protection of works in advance of each rainy season. All damages to construction site, works and materials, arising from operations not protected in accordance with the Specifications shall be repaired immediately by the Contractor at its expense. The Contractor shall also take all steps necessary to complete drainage in advance of each winter rainy season.

Notwithstanding approval of the intended method of working, the Contractor shall at all times be responsible for constructing the works in accordance with the Specifications, the Design and drawings.

### PREVENTION OF POLLUTION

The Contractor shall ensure that its activities do not result in any contamination of land or water by polluting substances.

### ENVIRONMENTAL CONSIDERATIONS

The following environmental protection measures shall be observed during the execution of the construction of the works:

- Demolition material - The Contractor will be responsible for environmentally safe disposal of any material resulting from the demolition and other site materials with approval from the relevant local Authorities at a designated licensed disposal facility.

- Air pollution - Construction may give rise to dust and construction equipment exhaust emissions. Due note shall be taken of the proximity of residential housing to the works. The normal health and safety controls will be required to safeguard the residential and passing population.

- Noise pollution - Construction works may cause annoyance caused by noise. The normal health and safety controls will be required to safeguard the residential and passing population.

- Maximum noise levels - During construction works the Contractor shall comply with the local and national requirements. The Contractor shall be legally responsible and financially liable to observe Serbian environmental legislation.

The noise levels shall be in accordance with the relevant Serbian noise environmental legislative.

Noise and disturbance shall be kept to the reasonable minimum as far as required for this project. The Contractor’s attention is drawn to the close proximity of some residential areas. All plant and tools used at such sites above or near ground level shall be silenced or of a silent type.

The Contractor shall take all necessary steps to ensure that its workmen carry out their duties in a quiet manner particularly when working at night.

- Pollution prevention - The Contractor shall not pollute or unnecessarily disturb lands, roads and other places on and around the Site. No trees or other vegetation shall be removed except to the extent necessary for the Works.

### AIR QUALITY

- The Contractor shall devise and arrange methods of working to minimize dust, gaseous or other air-borne emissions and carry out the Works in such a manner as to minimize adverse impacts on air quality.

- The Contractor shall utilize effective water sprays during the delivery and handling of materials when dust is likely to be created, and to dampen stored materials during dry and windy weather.

- Stockpiles of materials shall be sited in sheltered areas. Stockpiles of friable material shall be covered with clean tarpaulins, and sprayed with water during dry and windy weather. Stockpiles of material or debris shall be dampened prior to their movement, except where this is contrary to the Specification.

- Any vehicle transporting no coherent material shall not be loaded to a level higher than the side and tail boards, and shall be covered with a clean tarpaulin in good condition. The tarpaulin shall be properly secured and extend at least 300 mm over the edges of the side and tail boards.

- In periods of high wind, dust generating operations shall not be permitted within 200 m of residential areas having regard to the prevailing direction of the wind.

- Construction vehicles and machinery shall be kept in good working order and engines turned off when not in use. Appropriate measures shall be taken to limit exhaust emissions from construction vehicles, machinery and plant.

- An advance warning shall be given to potentially affected persons, so that some measures can be taken by them before commencement of works, especially before dismantling/demolition.

### NOISE

- The Contractor shall consider noise as an environmental constraint in its planning and execution of the Works.

- The Contractor shall take all necessary measures to ensure that the operation of all mechanical equipment and construction processes on and off the Site shall not cause any unnecessary or excessive noise, taking into account applicable environment requirements. The Contractor shall use all necessary measures and shall maintain all plant and silencing equipment in good condition so as to minimize the noise emission during construction works.

### MEASURES FOR DECREASING THE NEGATIVE ENVIRONMENTAL IMPACT

In order to mitigate negative environmental impact, the Contractor should propose necessary actions in its Environmental Management Plan (EMP), such as:

- to create adequate organisation for execution of construction works which shall comply with local construction regulations;

- to provide water sprinkling of the construction site

- to create organisation for control on the facilities storing fuel and lubricants and on the technical condition of the machines in order to avoid accidental oil spills;

- along the construction site, waste water should be treated and sedimentation tanks and oil separators should be placed if needed;

- to specify the quantity and type of waste and how its disposal is intended to be transported and removed from the site area;

- Measures for fast conservation of unfinished works at unfavourable conditions.

# MATERIALS

## GENERAL

- All materials used shall be of the best quality as specified and described in the Specification, Design, Drawings and the Bills of Quantities. Where in the Design Drawings and/or BoQ the products are brand named, this should be understood as supplemented by 'or equivalent'. These materials shall be procured from approved manufacturers or suppliers.

- The Contractor must secure the compliance with the Specification of materials or plant to be provided under this Contract before the supplier or manufacturer is proposed for approval to the Supervisor.

- The Contractor shall take into consideration the local climatic and other environmental conditions when selecting and proposing the materials. The quality of the material has to be confirmed by the attests and suppliers certificates, all according to TS and RS regulations.

- Whenever possible, the Contractor shall provide equipment of a similar nature from the same manufacturer, e.g. electric motors;

The Contractor shall note that particular attention will be paid to these requirements. In cases where the proposed equipment is not standardized with regard to manufacturer and type, the Contractor shall be required to provide conclusive technical justification; considerations of price alone will not be accepted.  Equipment and components that have not been standardized will not be accepted.

## ORIGIN

Certificates of origin have to accompany the products proving that supplies originate from an eligible country as stated in GC of the Contract.

## CONFORMITY OF MATERIALS

All materials implemented during construction shall be in compliance with the requirements of:

- Requirements of the local legislation;

- REGULATION (EU) No 305/2011 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 9 March 2011 laying down harmonized conditions for the marketing of construction products;

- The present Technical Specifications;

- Requirements of the design documentation;

- CONDITIONS FOR TAKING TECHNICAL PROTECTION MEASURES, Institute for the Protection of Cultural Monuments of Serbia.

All materials applied shall be accompanied with quality certificates to prove their concordance with the requirements set out in the design, the Specification and the Code for Civil Construction Works.

The Contractor shall make diligent efforts to procure the specified materials. Where, due to different reasons, the materials required by the Contract are not available, substitute materials may be used but with the prior approval by the Supervisor and the Designer.

# TECHNICAL REQUIREMENTS FOR EXECUTION OF WORKS

## TECHNICAL REQUIREMENTS

### PURPOSE OF THE TECHNICAL REQUIREMENTS

The purpose of the technical requirements is to provide quality performance of works to comply with technical regulations and standards. Therefore, the Contractor is obliged to adhere strictly to them and to perform all the works that are the subject of this project, in accordance with technical requirements, design documentation, accompanying drawings and Bill of Quantities.

In addition, technical requirements define the method of measurement. Therefore, bidders are required to include all costs for not separately analysed and measured items in the unit prices of the existing ones.

## GENERAL TERMS

### NOTICE OF COMMENCEMENT

The Contractor shall give a written notice to the Supervisor of its intention to commence works (Notice of Commencement).

Also, according RS law, Notice of Commencement must be submitted to competent Local and National Authority.

The works shall not be commenced until written approval has been received from the Supervisor.

### TECHNICAL SPECIFICATION FOR WORKS

Technical Specifications are an integral part of the Tender Documentation, and are annexed to the Works Contract.

The Contractor is fully familiar with all details of the provided design documentation, as well as with all local regulations, local standards (SRPS), common practice of trade and circumstances for their execution. Nevertheless, it is understood that, whenever local regulations, local standards (SRPS), or any common practice of trade, are subject to any interpretation, clarification, ambiguity, or dispute, a ruling by the Supervisor will prevail, always provided that such ruling will be fully in compliance with and will be based on the subject local regulations, local standards (SRPS), as well as in accordance with common practice of trade, and any such ruling by the Supervisors and subsequent instruction in that respect, will not constitute any ground for variation order and/or any additional payment.

Communication between the Contractor and the Beneficiary (also Designer), during the works will be carried out exclusively through the Supervisor.

All works must be carried out precisely and professionally. Prior to application, the Supervisor must examine all material and all his comments referring to material and quality of work will be obligatory for the Contractor. The agreed prices include all fully completed works and final products ready for use.

The Contractor will be responsible for any and all damages caused by the Contractor during any works, to any third party, structure, main building or adjacent buildings, and all repair works and compensations of any kind will be at the Contractor’s expense.

Prior to the commencement of the works, and also in the course of the execution of every work item, the Contractor will ask the Supervisor for any explanations and clarifications required, therefore, the Contractor will solely bear full material responsibility for all works not completed in accordance with the concept and details of this specifications.

The Contractor will be responsible to keep records on the progress of works in the measurement book and have it controlled and verified by the Supervisor.

Upon the completion of the works the Contractor will remove from the building site and other used areas all its tools, machinery, surplus material, etc. so as to have the site neatly arranged as defined in the investment-technical documentation, and all other areas restored in same condition as before the construction.

All construction works must be carried out under the conditions and in the manner prescribed by RS Law on Construction and Planning.

For all works, applicable Serbian regulations and standards shall prevail.

## TECHNICAL STANDARDS AND REGULATIONS

In accordance to these Technical Requirements the Contractor shall ensure that its performance incorporates the following key principles

- For all required works and services specified in this Tender Dossier, the relevant Serbian standards and codes of practice shall apply. In any case, if Serbian standards are more strict or dominant, they shall apply to replace other standards given or not in other parts of this document.

- For works and services where no relevant Serbian standards or codes of practice exist the latest European Standards and code of practice shall be applied.

- The proposed application of other standards and code of practice for certain works and/or services shall be such as to ensure equal or higher than specified quality and safety of works, and to facilitate operation, inspection, maintenance, repairs, lubrication and similar operations.

- In any case, National standards and code of practice have to be used for each service and work, accompanied with explanations, to demonstrate to the agreement of the Supervisor that application of these standards and code of practice shall give required quality, safety, functionality and durability of the completed works.

- The applicable version of any standard shall be that valid 28 days prior to the latest date for submission of tenders.

## MATTERS NOT COVERED BY THE STANDARDS

Any materials and workmanship not fully specified herein or covered by the Standards, Codes or Manuals shall be of such type and quality so as to produce a required quality of work. In such circumstance the Supervisor shall determine whether all or any of the materials offered or delivered to the site are suitable for use in the Works and the Supervisor’s decision in this respect shall be final and conclusive.

## CONTRACTOR'S EQUIPMENT

Details of all Contractors’ Equipment to be used in the execution of the Works shall be submitted to the Supervisor prior to its use.

The Supervisor's consent to use such Equipment will not be unreasonably withheld, but if, in the Supervisor's opinion, circumstances arise which make it desirable that the use of the said Equipment should be suspended either temporarily or permanently, the Contractor shall change the method of performing the work affected and it shall be deemed to have no cause for claims against the Supervisor on account of having to carry out the work by another method, nor it shall be deemed to have cause for claim if any order issued by the Supervisor results in the Contractor's Equipment having to stand idle for a period of any duration whatsoever or having to be removed.

## SUBCONTRACTED WORKS

The Contractor shall appoint subcontractors for the work for which the Contractor is not experienced, recognized or approved.

The Contractor shall submit for consent, the names of all proposed subcontractors and suppliers of special manufactured items with full details of the company, reference list and all other documentation needed for approval of the subcontractors and shall indicate the precise sections of the work for which each will be responsible.

The Contractor shall be solely responsible for the overall co-ordination of the Contract. Direct formal communication between its sub-contractors and the Supervisor will not be allowed.

## METHOD STATEMENTS

The Contractor shall provide, in writing, a description of the arrangements and methods it intends to apply for the execution of the Works.

Method Statements (MS) shall show in detail the methods proposed by the Contractor for carrying out the principal activities of construction in full safety. In particular, the Contractor shall indicate the resources (plant, personnel, materials) to be allocated, timing and sequencing, emergency/contingency measures, and any other information required to clearly detail the proposed methods. All necessary health and safety and environmental measures required shall be clearly indicated.

Flowcharts, sketches and drawings shall be included if necessary.

Proposed MS will be submitted to the Supervisor (also to the Beneficiary) for approval. The Supervisor (in cooperation with the Beneficiary's representative) will review and provide its comments within 10 days. The Contractor shall make final corrections (if any) and submit them to the Supervisor for final approval, 15 days before the commencement of relevant work. Written agreement shall be obtained before any work is commenced.

## PROVISIONAL TIME SCHEDULE

The Defects Notification Period (DNP) shall be twelve (12) months under the Contract and shall commence after completion of the Works. In accordance with the applicable legislation of the Republic of Serbia, the Contractor shall provide an additional warranty period of 1 (one) year for all works and materials, except for the roof tiles installed. For the roof tiles, the Contracting Authority requires a warranty period of 50 (fifty) years.

The duration and sequence of the various activities constituting the Works may be varied by the Contractor to suit its own proposals for carrying out the works, subject to the approval of the Supervisor, but no consideration will be given to any request by the Contractor to extend the Contract completion dates.

## STANDARDS ON THE SITE

The Contractor shall purchase and keep on Site at least one copy of each of the relevant Standards, Codes and Manuals or approved National Standards which are referred to in the Specification.  In addition, the Contractor shall keep on Site a copy of any other Standard, Code, Manual, or National Standard, which applies to materials supplied.

Copies of the standards shall be made available for reference at all times at the office of the Supervisor.

Should the Supervisor require an English or Serbian translation of any of the Standards or Manuals, the Contractor shall provide a translation within 7 days of receiving a written request from the Supervisor.

# TECHNICAL SPECIFICATIONS OF THE MOBILE FIRE EXTINGUISHING EQUIPMENT

## Hand-held portable fire extinguishing apparatuses using powder - Technical requirements

The apparatuses shall have cylindrical shape. The range of the jet must be minimum 3 m, and the quantity and pressure of the propellant gas must ensure equal powder ejection. The remaining powder in the tank must not exceed 10% of the original quantity. The apparatuses shall be operational at temperatures from – 20 to + 45oC. The free play of the units for activation of the apparatuses (button, handle, lever ...) must be 3  1 mm, and the hand wheel on the valve of the propellant gas bottle shall have free play at the angle of 30o. The direction of opening of the hand wheel shall be counter-clockwise. The apparatuses shall always be used in the upright position.

The apparatuses having filings over 3 kg (S-6, S-9, S-12) must have the discharge nozzle (with a mechanism for release and interruption of the powder jet) connected to the tank, by means of a flexible hose 800 mm long, and the discharge nozzle and the hose must be of materials that do not conduct electricity. The apparatuses type S shall be delivered filled (the powder and the propellant gas under pressure).

The apparatuses shall be filled with the powder up to 3/4. In smaller apparatuses, carbon dioxide shall be used as the propellant gas and, for bigger apparatuses, nitrogen. The allowable deviation in the filling of apparatuses with powder shall be:

- For apparatuses up to 3 kg:	  up to 3% ;

- For apparatuses over 3 kg:	 0.2 kg.

The allowable deviation in the quantity of gas under pressure in a steel bottle shall be  2%, and the allowable deviation in the pressure of gas in the tank, which is to be under constant pressure, shall be  10% of the indicated operating pressure at the temperature of 20oC.

Technical data for hand-held portable fire extinguishing apparatuses using powder:

| Apparatus | Powder content kg | Gas content g | Gross weight kg | Action time s | Jet range m | Operating pressure bar | Test pressure bar | Safety valve bar |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-0,5 | 0.5 | 10 | 1.6 | 4-5 | 3–4 | 10–12 | 20-25 | 16 |
| S-1 | 1 | 20 | 2.5-3.2 | 7-8 | 4-5 | 12 | 20-25 | 16 |
| S-2 | 2 | 36-40 | 4.2-5.3 | 8-12 | 4-5 | 12 | 20-25 | 16 |
| S-3 | 3 | 50-60 | 5.5-7 | 12-17 | 4-5 | 12-15 | 20-25 | 16 |
| S-6 | 6 | 120-130 | 11-12 | 12-15 | 4-6 | 12-14 | 22-25 | 16-19 |
| S-9 | 9 | 160-200 | 15.7 | 20-22 | 4-6 | 12-14 | 22-25 | 16-19 |
| S-12 | 12 | 200-250 | 18-19 | 25-28 | 4-6 | 12-15 | 22-25 | 16-19 |

There are various workmanships (versions) of the apparatus type S, but all of them function in the same way in principle.

VOLUME 3

Section 2

DATA ON THE FACILITY AND LOCATION

| type of facility: | semi-detached buildings |  |
| --- | --- | --- |
| type of works: | adaptation, reconstruction and extension |  |
| predominant facility category: | “V” |  |
| classification of individual parts of the facility: | share in total facility area (%): | classification code: |
|  | 100% | facility 1: 127410 (V) – other buildings not otherwise classified for firefighters usable area: 275.73 m² gross construction area: 357.00 m² |
|  | 100% | facility 2: 127410 (V) – other buildings not otherwise classified for firefighters usable area: 433.77 m² gross construction area: 570.00 m² |
|  | 100% | facility 3: 127410 (V) – other buildings not otherwise classified for firefighters usable area: 277.37 m² gross construction area: 328.00 m² |
| name of spatial or urban plan: | General Regulation Plan of the Horgoš Settlement (Official Gazette of the Municipality of Kanjiža: 8/2011 and 12/2011) |  |
| location: | Horgoš |  |
| cadastral parcel number/list of cadastral parcels and cadastral municipality of facilities/works subject to the works: | 940 k.o. Horgoš |  |
| cadastral parcel number/list of cadastral parcels and cadastral municipality over which infrastructure connections subject to the request pass: | 16761 k.o. Horgoš |  |
| cadastral parcel number/list of cadastral parcels and cadastral municipality on which above-ground parts of linear facilities/connection lines related to land surface (entry and exit points, inspection shafts, etc.) subject to the request are located: | 16761 k.o. Horgoš |  |
| cadastral parcel number/list of cadastral parcels and cadastral municipality on which existing lines in conflict with the planned works are located: |  |  |
| cadastral parcel number/list of cadastral parcels and cadastral municipality to which existing lines are relocated (if relocation is subject to the request): |  |  |
| cadastral parcel number/list of cadastral parcels and cadastral municipality on which existing facilities to be removed are located: |  |  |
| cadastral parcel number/list of cadastral parcels and cadastral municipality on which the connection or access to a public traffic area is located: | 16761 k.o. Horgoš |  |
| INFRASTRUCTURE CONNECTIONS: |  |  |
| Electric power distribution network: | Existing connection, ED number: 229 82761 |  |
| Connection to (installation, network) | The facility is supplied from the existing low-voltage distribution network via an existing cable, while the existing MRO is located on the exterior wall of facility no. 1. Via the existing cable, PP00 4x10 mm², the MRO supplies the GRO, from which all sub-distribution cabinets are supplied. Secondary measurement of electricity consumption is located in the GRO cabinet, separately for each sub-distribution. The GRO is located on the wall of facility no. 1 in the dry entrance area. The power supply system is TN-C-S. Separate N and PE conductors are led from the GRO. |  |
| Total capacity: | - |  |
| Type of connection: | Existing |  |
| Required capacities for different purposes (classified by entrances): | - |  |
| Required capacities for common consumption (classified by entrances): | - |  |
| Data on connections of existing facilities on the parcel/parcels (if any): | Existing connection, ED number: 229 82761 |  |
| Missing infrastructure in accordance with Holders of Public Authorizations conditions: | - |  |
| Atypical consumers: | - |  |
| Other infrastructure: | - |  |
| connection to drinking water and sewerage: | Existing connection, water meter number: 2555800. The existing drinking water and sanitary sewer connection meets the needs of the planned capacities. |  |
| connection to natural ga: | Existing connection, consumer number: 3665447D On the pole of the subject facility, according to the attached graphic documentation, a KMRS G-6T with a flow measurement range of up to 10 m³/h (inlet pressure pu = 2.5 bar, outlet pressure pi = 22 mbar), a standard solution of JP “SRBIJA GAS”, is installed. A complete metering and regulation set is installed, consisting of: a) pressure regulator, maximum inlet pressure up to 4 bar and set outlet pressure of 22 mbar, with a maximum gas flow of 10 Sm³/h, SOV opening pressure 34 mbar, connection dimensions DN25, b) diaphragm gas meter, size G-6T, measuring range from 0.04 to 10 Sm³/h, maximum operating pressure 500 mbar, c) gas filter, dimensions DN25 PN6 |  |

LOCATION CONDITIONS:

| Location conditions: | ROP: ROP-KAN-34302-LOCH-2/2023 date: 8.11.2023. |
| --- | --- |

CONDITIONS OBTAINED OUTSIDE THE UNIFIED PROCEDURE:

| Conditions: | number: date: |
| --- | --- |
|  | number: date: |

CONSENTS:

| Issued consents: |  |
| --- | --- |
|  | number: date: |

BASIC DATA ON THE FACILITY AND LOCATION:

| facility dimensions: | total area of the parcel/parcels: | 1350 m² |
| --- | --- | --- |
|  | total GROSS built-up area after completion of planned works: facility 1: 357m² facility 2: 570m² facility 3: 13m² facility 4: 328m² | 1268 m² |
|  | total GBA built-up area: total above-ground GBA area constructed: facility 1: 350m² facility 2: 270m² facility 3: 13m² facility 4: 160m² | 793 m² |
|  | total NET area: | 986,87 m² |
|  | GROSS ground floor area: | 819 m² |
|  | land area under the facility/coverage (increased due to demit facade): | 60,67% |
|  | number of storeys (above-ground and underground levels) FACILITY NO. 1: | P |
|  | number of storeys (above-ground and underground levels) FACILITY NO. 2: | P+1 |
|  | number of storeys (above-ground and underground levels) FACILITY NO. 4: | P+1 |
|  | facility height (eaves, ridge, recessed storey, etc.) according to location conditions FACILITY NO. 1: | ridge: 7,77m eaves: 4,45m |
|  | facility height (eaves, ridge, recessed storey, etc.) according to location conditions FACILITY NO. 2: | ridge: 7,63m eaves: 6,42m |
|  | facility height (eaves, ridge, recessed storey, etc.) according to location conditions FACILITY NO. 4: | ridge: 7,66m eaves: 6,42m |
|  | absolute elevation (eaves, ridge, recessed storey, etc.): | ridge: +95,06mnv eaves: +91,74mnv |
|  | storey height: | 2,55m; 2,60m; 2,80m, 3,00m i 4,00m |
|  | number of functional units/number of apartments: | 3 functional units |
|  | number of parking spaces: | 0 |
| special parts of the facility: | number of apartments: | 0 |
|  | number of commercial spaces: | 0 |
|  | number of garages/garage spaces: | 3 |
|  | number of parking spaces: | 0 |
| facility materials: | facade material: | bavalit |
|  | ridge orientation: | north–west |
|  | roof slope: | 5°, 8°, 35° |
|  | roof material: | folded Twiston roof tiles and metal roof tiles with stone coating |
| percentage of green areas: |  | 30% |
| site coverage index: |  | max 60% |
| floor area ratio: |  | 58,4% |
| heating method: | Natural gas from the existing connection. |  |
| other facility characteristics: | - |  |
| final calculated value of the facility: | provided in As-Built Design |  |

LOCATION DATA

The number of the subject construction parcel is: 940 c.m. Horgoš, located at the address Bela Bartok Street No. 14, in the settlement of Horgoš. According to the valid General Regulation Plan of the Horgoš settlement, the parcel belongs to the central facilities of general public interest of the Horgoš settlement. In the vicinity of the parcel there are family residential buildings.

The total area of the parcel is 1,350 m². There are 4 buildings constructed on the parcel according to the following reference numbers on the cadastral plan copy:

1 – fire station (construction area 350 m²);

2 – building for firefighters (construction area 270 m²);

3 – fire observation tower (construction area 13 m²);

4 – facility for storage of firefighting equipment (construction area 160 m²).

The total built-up area on the parcel is 793 m², which represents 58.4% site coverage of the buildings on the parcel. All existing buildings on the parcel are ground-floor buildings. After the execution of the planned works, facilities No. 2 and 4 will have storeys G+1. Facilities No. 1 and 3 will not change their number of storeys after the execution of the works.

After the execution of the planned works, the GROSS DEVELOPED CONSTRUCTION AREA OF THE BUILDINGS WILL BE AS FOLLOWS:

1 – fire station, storeys ground floor (G) 357 m²;

2 – building for firefighters, storeys ground floor and one floor (G+1) 570 m²;

3 – fire observation tower – tower, unchanged 13 m² – not subject to technical documentation;

4 – facility for storage of firefighting equipment, storeys ground floor and one floor (G+1) 328 m².

Vehicular and pedestrian access to the parcel is from Bela Bartok Street, which has been constructed.

FUNCTION

FACILITY NO. 1 – FIRE STATION

In functional terms, Facility No. 1 is intended as a fire station. It is used for gathering and conferences of firefighters, as well as for changing clothes before and after firefighting operations. After the planned works, this function will not change. The planned works include adaptation of interior rooms and installations, complete reconstruction of the roof structure, and energy rehabilitation of the facility with replacement of exterior and interior joinery, as well as works classified as regular maintenance of the facility. After completion of the roof reconstruction works, the facility will have a ridge height increased by 0.75 m compared to the existing height, amounting to 6.95 m. The number of storeys of Facility No. 1 will not change. The construction area increases to 357 m² due to the installation of a demit facade, and the usable area after adaptation of interior rooms is 276.50 m².

According to the Rulebook on Classification of Buildings (Official Gazette of the RS No. 22/2015), the facility belongs to other buildings, not otherwise classified as other buildings for firefighters, which is a high-complexity facility of category “V” with classification number 127410.

Facility No. 1 on the parcel is defined by graphical annexes with marked building and regulation lines. The building line of Facility No. 1 coincides with the regulation line, which is also shown in the graphical annexes.

All rooms will retain the same purpose as in the existing condition.

The ground floor layout of the facility is close to an “L” shape with rough external dimensions: 21.43 m x 11.47 m and 16.21 m x 8.13 m.

The vertical dimension of the facility is ground floor (G).

The ground floor level is raised by 24 cm above the ground level.

The clear height of rooms on the ground floor is 3.00 m.

The ridge height of the facility is 7.77 m measured from ground level.

The terrain slopes toward the interior of the parcel. The height difference between the sidewalk at the street and the ground level in front of Facility No. 4 is about 40 cm, which is also shown in the attached Cadastre topographic plan.

The roof structure of the fire station is a complex three-pitched structure with roof slopes of 35° and 36°. The roof surface is covered with folded roof tile.

Planned period of implementation: The object No. 1 – The demolition works, earthworks, masonry works, carpentry works, roofing and sheet metal works must be completed within 52 working days at the latest.

FACILITY NO. 2 – BUILDING FOR FIREFIGHTERS

In functional terms, Facility No. 2 is intended as a building for firefighters. The facility is used for gathering and firefighter conferences.

After planned works, Facility No. 2 will obtain one additional floor. On the ground floor, the interior rooms will be reorganized to form the following rooms: sanitary block for persons with disabilities, female and male sanitary blocks, changing room, conference hall, kitchen, bar, storage room, corridor with staircase, and a recreation hall for firefighters. On the added part, on the first floor, a corridor with staircase, a multipurpose room for employees, and 8 rooms with private bathrooms will be formed, enabling overnight stays for firefighters with a total of 25 beds. Since an extension of Facility No. 2 is planned, it will also receive a new roof structure with a single gable roof. On the ground floor of Facility No. 2, adaptation of interior rooms and installations is planned, energy rehabilitation of the facility with replacement of exterior and interior joinery, as well as works classified as regular maintenance of the facility. After completion of the works with construction of the additional floor, the facility will have a ridge height increased by 2.78 m compared to the existing height, amounting to 4.85 m, and the total ridge height will be 7.63 m. The construction area and number of storeys of Facility No. 2 will change. The existing construction area is 270 m², and after completion of the works it will have a gross construction area of 570 m². The ground floor construction area increases to 285 m² due to installation of a demit facade on the exterior walls. The usable area after completion of the works will be 433.77 m² according to SRPS.U.C2.100:2002. The existing usable area is 229.14 m², meaning that both the usable and construction areas will change.

According to the Rulebook on Classification of Buildings (Official Gazette of the RS No. 22/2015), the facility belongs to other buildings, not otherwise classified as other buildings for firefighters, which is a high-complexity facility of category “V” with classification number 127410.

Facility No. 2 on the parcel is defined by graphic enclosures with marked building and regulation lines. The building line of Facility No. 2 is recessed toward the interior of the parcel by 28.35 m measured from the regulation line, which is also shown in the graphical annexes.

The extended facility will have the following number of storeys: ground floor and one floor (G+1).

The ground floor layout of the facility is of a regular rectangular shape with external dimensions including the demit facade: 7.30 m x 38.02 m.

The ground floor level is raised by 10 cm above ground level.

The clear height of rooms on the ground floor is 2.55 m and 2.80 m, and on the first floor it is 2.80 m.

The ridge height of the facility is 7.63 m measured from ground level.

The terrain slopes toward the interior of the parcel. The height difference between the sidewalk at the street and the ground level in front of Facility No. 4 is about 40 cm, which is also shown in the attached Cadastre topographic plan.

The roof structure of Facility No. 2 is a single gable roof structure with a roof slope of 8°. The roof surface will be covered with metal roof tiles with stone coating.

Planned period of implementation:

The demolition of the roof and interior walls must be completed within 72 working days at the latest.

Concrete and masonry works must be completed within 113 working days at the latest.

Carpentry and roofing work must be completed within 142 working days at the latest.

Interior works, exterior decoration and all craft works must be completed no later than the expiry of the agreed deadline, i.e. in the period from 142 to 272 working days.

FACILITY NO. 4 – FACILITY FOR STORAGE OF FIREFIGHTING EQUIPMENT

In functional terms, Facility No. 4 is intended as a garage and storage for various firefighting tools and equipment.

After the planned works, Facility No. 4 will obtain one additional floor. One part of the ground floor will be reorganized in order to form a staircase enabling access to the first floor. On the first floor, a room will be formed for training firefighters to jump from heights through windows, and on the roof surface an area will be formed for jumping from a height of 7.00 m.

Since an extension of Facility No. 4 is planned, it will also receive a new roof structure with a single gable roof, and the roof structure will be completely reconstructed. On the ground floor of Facility No. 4, renovation of the floor surface is planned, replacement of exterior and interior joinery, as well as works classified as regular maintenance of the facility. After completion of the works with construction of the additional floor, the facility will have a ridge height increased by 0.46 m compared to the existing height, amounting to 7.20 m, and the total ridge height will be 7.66 m. The construction area and number of storeys of Facility No. 4 will change. The existing construction area is 160 m², and after completion of the works it will have a gross construction area of 328 m², while the usable area after completion of the works will be 278.98 m². The existing usable area is 148.06 m², meaning that both the usable and construction areas will change. The ground floor construction area increases to 164 m² due to installation of a demit facade on the exterior walls. The number of storeys changes from a ground-floor facility to a facility with ground floor and one floor.

According to the Rulebook on Classification of Buildings (Official Gazette of the RS No. 22/2015), the facility belongs to other buildings, not otherwise classified as other buildings for firefighters, which is a high-complexity facility of category “V” with classification number 127410.

Facility No. 4 on the parcel is defined by graphic enclosures with marked building and regulation lines. The building line of Facility No. 2 is recessed toward the interior of the parcel by 52.80 m measured from the regulation line, which is also shown in the graphical annexes.

The extended facility will have the following number of storeys: ground floor and one floor (G+1).

The ground floor layout of the facility is of a regular rectangular shape with external dimensions: 12.13 m x 13.20 m.

The ground floor level is raised by 3 cm above ground level.

The clear height of rooms on the ground floor is 4.00 m, and on the first floor it is 2.60 m.

The ridge height of the facility is 7.66 m measured from ground level.

The terrain slopes toward the interior of the parcel. The height difference between the sidewalk at the street and the ground level in front of Facility No. 4 is about 40 cm, which is also shown in the attached Cadastre topographic plan.

The roof structure of Facility No. 4 is a single gable roof structure with a roof slope of 5°. The roof surface will be covered with metal roof tiles with stone coating.

EXISTING CONDITION

Structure and materials

There is no technical documentation on the foundations of Facilities No. 1, 2 and 4. The vertical structural system of the facilities consists of masonry walls and reinforced concrete columns.

The ground floor walls of Facility No. 1 are constructed of bricks with thicknesses of 25 and 38 cm without reinforced concrete ring beams. The ground floor walls of Facilities No. 2 and 4 are constructed of standard-format bricks with reinforced concrete columns and ring beams, with lightweight prefabricated floor elements (FERT) and reinforced concrete slabs. The walls are plastered on both sides with extended cement mortar and painted with lime paints. Floors are concrete without waterproofing. Wall waterproofing has not been executed, so it needs to be installed in Facilities No. 1 and 2. Floors in Facilities No. 1 and 2 are finished with ceramic tiles. In Facility No. 4, the floor is reinforced concrete. Interior and exterior joinery in Facility No. 1 is wooden, while in Facilities No. 2 and 4 it consists of metal frames, glazed with ordinary glass.

The storey height of the ground floor is approximately 260 cm. The ceiling above the ground floor in Facility No. 1 is executed as a wooden structure – wooden ceiling – with boards and clay in the attic area. In Facility No. 2, the ceiling is a TM structure, and in Facility No. 4 it is a cross-reinforced concrete slab.

The roof structure of all facilities is wooden. In Facility No. 1, it is a complex three gable roof structure on frames. The load-bearing elements of the roof covering are wooden rafters, from which the load is transferred to the wooden roof structure consisting of purlins and columns. The roof covering consists of folded roof tiles on battens. The roof covering on Facilities No. 2 and 4 consists of corrugated asbestos-cement sheets. The roof slopes are 35° and 12°.

Atmospheric water from the roof surfaces is collected by appropriate gutters. A sidewalk has been constructed around the facility, which diverts atmospheric water away from the exterior walls. The complete vertical and horizontal gutter system will be replaced.

NEW CONDITION

Structure and materials

Existing floor finishes in Facilities No. 1, 2 and 4 are replaced with new finishes according to the purpose of the rooms, all in accordance with the graphic enclosures and the bill of quantities. Under the ceramic tiles, thermostable concrete and rigid styrofoam with thickness d = 3 cm are installed. The final floor finish consists of ceramic tiles set in adhesive for ceramic tiles. A ceramic tile skirting with a width of 10 cm is installed on the walls. The ±0.00 level does not change compared to the existing condition, nor does the interior height of the ground floor rooms.

All joinery is replaced, except on Facility No. 1 on the street front, where the existing wooden windows are refurbished; all other joinery is replaced with PVC profiles with insulated glazing and accompanying PVC drip edges and window sills. New openings are also formed on the facade walls, all in accordance with the graphical annexes.

Newly built walls are constructed of YTONG blocks or similar with thicknesses of 15, 25 and 30 cm, depending on the purpose of the walls and rooms. Newly formed walls are plastered with extended cement mortar. New lintels above doors and windows are made of reinforced concrete and executed on site with the necessary shoring.

At wall junctions, vertical ring beams are executed in the wall thickness, and at the level of the inter-storey structure, a horizontal ring beam is executed.

The ceiling structure above the first floor in Facility No. 2 is a fine-ribbed FERT structure, and in Facility No. 4 the inter-storey structure is also a FERT slab with a reinforced concrete topping slab 4 cm thick.

Walls and ceilings are plastered with extended cement mortar.

Floors on the upper storey in Facility No. 2 are finished with non-slip ceramic tiles set in adhesive on thermostable concrete with thickness d = 8 cm, with a thermo-reflective foil, rigid Styrofoam with thickness d = 3 cm, and PVC foil on a reinforced concrete slab. Walls and ceilings are skim-coated and painted in two coats.

The roof structure above Facility No. 1 is a complex three-pitched structure. The roof slope is 35°. A roofing membrane with counter-battens is installed on the rafters, followed by battens for folded roof tiles. The eaves are clad with tongue-and-groove boards, which are painted with a translucent stain. The roof structure above Facilities No. 2 and 4 is a lightweight wooden structure for metal roof tiles with stone coating, Korra or similar manufacturer, ruby colour or similar.

The ceiling structure in Facilities No. 1 and 2 is insulated with mineral wool for thermal insulation of the rooms below.

Thermal insulation of facade walls (existing and new walls) is carried out with a demit system with a thickness of 10 cm and finished with decorative facade plaster – bavalit or similar– in a colour selected by the Investor and the designer.

Suspended gutters, vertical downpipes, valleys and flashings made of galvanized sheet metal are installed in accordance with the design documentation. Visible wooden roof elements are painted with a translucent stain.

EXTERNAL ARRANGEMENT OF THE FACILITY

It is planned that the vehicular access for special vehicles, which are firefighting vehicles, paths and parking areas within the parcel, and protective sidewalks around the facility will be constructed of asphalt surfaces with the required substructure. Pedestrian paths will be constructed of brushed concrete, over which decorative paving will be installed using materials of texture and colour selected by the Investor.

ELECTRICAL INSTALLATIONS

There is an existing electricity connection on the subject parcel, and a new connection is not required, but an increase in electrical capacity is required.

Existing meter number: ED no: 229 82761.

Considering the classification of evacuation characteristics BD1 and building structure CB1, cables and conductors of types PP-Y and N2XH, J-Y(St)Y, SFTP, RG59 B/U have been selected for all power circuits, and fire-resistant cables NHXHX FE180/E90 for safety circuits.

Throughout the facility, measures are provided to prevent the spread of fire by using fire-resistant cables and fire-resistant sealing of penetrations and coatings of installations at the boundaries of fire compartments.

Power supply of the facility: The facility is supplied from the existing low-voltage distribution network via an existing cable, while the existing Metering Cabinet is located on the exterior wall of Facility No. 1. Via the existing cable PP00 4x10 mm², the Metering Cabinet supplies the Main Distribution Board (MDB), from which all sub-distribution cabinets are supplied. Secondary measurement of electricity consumption is located in the MDB cabinet, separately for each sub-distribution. The MDB is mounted on the wall of Facility No. 1 in the dry entrance area. The power supply system is TN-C-S. Separate N and PE conductors are routed from the MDB.

HYDROTECHNICAL INSTALLATIONS

WATER SUPPLY AND SEWERAGE

A public water supply and sewerage network has been constructed in the subject street. The facilities have existing connections to this network. Atmospheric water is managed by dispersion and infiltration into the surrounding free area.

Water meter number: 2555800.

The facility is supplied with water from the municipal water supply system. The design provides for reconstruction of the existing connection. A connection to the public water supply network is planned via a polyethylene pipe with an outer diameter of 90 mm. Downstream of the connection, the water line branches into two branches. One branch is intended for firefighting water, where the consumed water is measured by a DN50 water meter, while the other branch is intended for sanitary water, where the consumed water is measured by a DN25 water meter. Both water meters are located in a manhole in front of the facility.

Used sanitary water is collected and discharged into the public sewerage network via a DN160 connection.

WATER SUPPLY INSTALLATIONS: The water demand of the facilities amounts to 1.3 l/s. Internal pipelines are made of Geberit Mepla multilayer aluminium-plastic pipes (PE-RT Type-II/Al/PE-RT Type-II) manufactured in accordance with EN ISO 21003-2:2008 and EN ISO 21003-3:2008, connected with “press” fittings made of PVDF or bronze, for floor distribution of sanitary cold and hot water (operating pressure up to 10 bar and temperature up to 70°C). Pipes are supplied in straight lengths or coils. Fixing shall be performed at minimum distances of 1.50 to 2.00 m. All installed pipes must be fully wrapped along their entire length with suitable insulation or anti-condensation material and pressure-tested. All nodes have the necessary shut-off valves to ensure proper water supply. All main branches have shut-off valves with drain outlets.

SEWERAGE: The total flow rate of sanitary wastewater from the facility amounts to 3.88 l/s. Upper and lower distribution of the internal sanitary sewerage is planned using PIPELIFE MASTER 3+ three-layer composite pipes reinforced with mineral fibres, made of plastic without halogens, cadmium and heavy metals, with reinforced pipe walls, minimum ring stiffness of 4 kN/m² (SN4). The smooth, reinforced inner layer of PP-C (polypropylene copolymer) is resistant to high temperatures up to 95°C. The stable, sound-insulated middle layer is made of PP-MV (polypropylene reinforced with mineral fibres). The outer layer is made of low-temperature-resistant polypropylene PP-C (polypropylene copolymer with snowflake marking). Integrated socket with factory-installed seal, tested and monitored in accordance with EN 1451. Sockets and pipe ends form a transition without irregularities that disturb flow. Pipes have clearly marked lengths with graduations every 5 mm. Fittings have external longitudinal reinforcement of the seal seat, positioned circumferentially at an angle of 60° for easier handling during installation. The seal seat is rectangular, with sharp edges preventing seal displacement during assembly. The fitting has an external marking indicating the maximum permissible insertion depth into the socket (muff).

Connections for all piping materials must be standard and watertight. Planned sewer ventilation stacks are 110 mm in diameter and terminate above the roof with ventilation heads. On stacks that do not terminate above the roof, a pipe air admittance valve must be installed at locations as shown in the graphic documentation. Around the ventilation head on the roof, protective galvanized sheet metal must be installed to prevent leakage. The entire sewer system must be constructed with a slope toward the outlet in accordance with the given sections. Sewer stacks are connected in the basement and routed along the ceiling.

MECHANICAL INSTALLATIONS

A public gas distribution network has been constructed in the street in question. The facilities have existing connections to this network.

Consumer code: 3665447D.

INTERNAL GAS INSTALLATIONS – EXISTING CONDITION

CONNECTION GAS PIPELINE (1-4bar)

The subject facility is already connected to the existing street distribution gas pipeline made of polyethylene, located in Bartok Bela Street. The existing connection gas pipeline will not be changed.

MEASURING AND REGULATION STATION

On the pole of the subject facility, according to the attached graphic documentation, a Domestic Measuring and Regulating Set G-6T with a flow measurement range of up to 10 m³/h (inlet pressure pu = 1–4 bar, outlet pressure pi = 22 mbar), a standard solution of JP “SRBIJA GAS”, is installed.

A complete metering and regulation set is installed, consisting of:

– a pressure regulator, maximum inlet pressure up to 4 bar and set outlet pressure of 22 mbar, with a maximum gas flow of 10 Sm³/h, SOV opening pressure 34 mbar, connection dimensions DN25,

– a diaphragm gas meter, size G-6T, measuring range from 0.04 to 10 Sm³/h, maximum operating pressure 500 mbar,

– a gas filter, dimensions DN25 PN6.

All equipment in the Domestic Measuring and Regulating Set meets the design and manufacturing criteria according to the Rulebook on Technical Norms for Internal Gas Installations (Official Gazette of the FRY No. 20/1992, No. 33/1992).

INTERNAL GAS INSTALLATION

The internal gas installation rises from the Domestic Measuring and Regulating Set via existing steel pipes DN20 to a height of 3 m and is routed along the wall of the facility entrance and connected to consumers according to the graphic documentation. Consumers include nine gas heaters and two gas boilers. A gas shut-off valve is installed in front of each gas consumer. Gas valves are galvanically bridged. The entire gas pipeline of the subject facility represents a galvanically continuous medium connected to the common grounding system.

6.5.1.2 INTERNAL GAS INSTALLATIONS – NEW CONDITION

Existing gas pipes DN20 and DN15 are dismantled together with the gas heater and all accompanying elements. The existing metering and regulation station MRS-G6T is retained, as it meets the newly designed criteria. The project boundary is the outlet gas valve DN25 after the MRS. The newly designed gas pipeline is low-pressure, 22 mbar.

The newly designed gas pipeline (22 mbar) CuØ28x1.5 after the gas valve behind the MRS is lowered to ground level and via a transition piece DN32–PEØ40 transitions into a polyethylene pipe. The gas pipeline is then routed underground at a depth of 0.8 m below the green area. The underground gas pipeline is divided into two branches with dimensions PEØ32x3 mm.

One underground branch PEØ32x3 mm is routed to Facility No. 1 and exits the ground. After the transition piece, the copper gas pipeline CuØ22x1 rises to a height H = 1.2 m and branches into two branches. One branch CuØ15x1 enters the kitchen, where it is connected to consumer No. 3, i.e. a gas stove with a power of 5 kW. The other branch CuØ22x1 is routed to the changing room and connected to consumer No. 1, i.e. a wall-mounted gas boiler type Baxi Luna Platinum +1.24 GA with a power of 24 kW or equivalent.

The second underground branch PEØ32x3 mm is routed to Facility No. 2. After the transition piece, the copper gas pipeline CuØ22x1 rises to a height H = 0.8 m and branches into two branches. One branch CuØ22x1 enters the facility and is connected to consumer No. 2, i.e. a wall-mounted gas boiler type BAXI Luna Duo-Tec MP+1.50 with a power of 45 kW or equivalent. The other branch CuØ15x1 is routed along the wall of the facility to the kitchen, where it enters the facility and is connected to consumer No. 4, i.e. a gas stove with a power of 5 kW.

A gas shut-off valve is installed in front of each gas consumer and must be galvanically bridged. The entire gas pipeline of one facility must represent a galvanically continuous medium connected to the common grounding system.

After completion of the works, a preliminary pressure test must be carried out at a pressure of 1.0 bar using air or inert gas (except oxygen). The test pressure must not decrease for a period of at least 10 minutes. The main tightness test is carried out at a pressure of 110 mbar, and the test pressure must not decrease for a period of at least 10 minutes. All other requirements are in accordance with the given technical conditions and documentation.

Construction phasing

The investor plans to execute the subject construction and construction craft works in one phase, depending on financial capabilities.

Additional information

All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

9.3. BILL OF QUANTITIES AND COST ESTIMATE

CONSTRUCTION AND CONSTRUCTION CRAFT WORKS

INVESTOR: MUNICIPALITY OF KANJIŽA, Kanjiža, Main Square No. 1 FACILITY: Reconstruction and adaptation of Facility No. 1 – Horgoš Fire Station PLACE OF CONSTRUCTION: Horgoš, Bartok Bela Street No. 14.